RAIN EVENT MONITORING MODULE

Implementation Appendix

Trigger Thresholds  •  Station Hierarchy  •  Override Rules  •  Outage Fallback

| Prepared by: | Brian Lewis, Chief AI Director — BlackRock AI |
| --- | --- |
| Date: | April 3, 2026 |
| Version: | 1.0 — Initial Implementation Specification |
| Status: | Approved Concept — Pending Operational Lockdown |
| Legal Privilege: | Attorney-Client Work Product via Steven R. Ruby, Esq. |
| Resolves: | Roadmap Tasks 2.56, 2.57, 2.58 |

This appendix locks down the four operational controls required to move the Rain Event Monitoring Module from concept to implementation: trigger thresholds per outfall, weather station hierarchy with fallback rules, supervisor override documentation requirements, and API/data outage standard operating procedures.

# A. Trigger Thresholds per Outfall

Every rain-event outfall must have a defined trigger standard in the system. No outfall may be left with a generic or undefined threshold.

## A.1 Trigger Definition Fields

Each outfall flagged as a rain-event monitoring point shall have the following trigger parameters stored in the platform. These fields are added to the sampling_schedules table as a JSONB column (rain_event_trigger) for outfalls where frequency_code = 'rain_event'.

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| rainfall_threshold_inches | decimal(4,2) | 0.50 | Minimum rainfall accumulation in a 24-hour period to qualify as a triggering event |
| trigger_window_hours | integer | 24 | Time window over which rainfall is measured against threshold |
| discharge_required | boolean | true | Whether visible discharge at the outfall is required in addition to rainfall threshold |
| field_confirmation_required | boolean | true | Whether station data alone triggers sampling or a field tech must confirm discharge before collection |
| max_sample_delay_hours | integer | 24 | Maximum hours after event trigger within which the sample must be collected |
| recurrence_interval_years | decimal(4,1) | null | If set, the storm must meet or exceed this return period to qualify (used for exemption claims, not routine triggers) |

## A.2 Current WV Rain-Event Outfalls

The following 5 WV outfalls are currently designated as rain-event monitoring points per the WV Self-Sampling Operations Manual. Trigger parameters shown are defaults pending permit-specific review by the Environmental Manager.

| Permit | Outfall | Threshold | Window | Discharge Req. | Field Confirm | Max Delay |
| --- | --- | --- | --- | --- | --- | --- |
| TBD from Matrix | Rain-001 | 0.50" | 24 hrs | Yes | Yes | 24 hrs |
| TBD from Matrix | Rain-002 | 0.50" | 24 hrs | Yes | Yes | 24 hrs |
| TBD from Matrix | Rain-003 | 0.50" | 24 hrs | Yes | Yes | 24 hrs |
| TBD from Matrix | Rain-004 | 0.50" | 24 hrs | Yes | Yes | 24 hrs |
| TBD from Matrix | Rain-005 | 0.50" | 24 hrs | Yes | Yes | 24 hrs |

ACTION REQUIRED: Environmental Manager must review each rain-event outfall’s permit and confirm or adjust the default trigger parameters. The Sampling Matrix (when received from Tom Lusk) will provide the definitive outfall-to-permit mapping.

## A.3 Trigger Logic Sequence

The system evaluates rain events using this exact sequence:

- NOAA CDO automated pull records daily precipitation at assigned station

- System compares reading against each linked outfall’s rainfall_threshold_inches within trigger_window_hours

- If threshold met → system generates RAIN_EVENT_ALERT notification to Supervisor role

- Supervisor reviews alert and either ACTIVATES (creates sampling_calendar entries) or DISMISSES (with mandatory reason code)

- If field_confirmation_required = true, dispatch includes instruction for tech to confirm visible discharge before collecting

- If tech arrives and no discharge is present, they record NODI code “C” per standard No Discharge protocol — this still counts as a completed response

- System timestamps every step and links all records (precipitation_reading → precipitation_event → sampling_calendar → sampling_event → lab_results)

# B. Weather Station Hierarchy & Fallback

Every site must have a primary and secondary station assignment. The system will not accept a site configuration with only one station.

## B.1 Station Assignment Rules

| Rule | Requirement |
| --- | --- |
| Primary station distance | Must be within 15 miles of the site centroid. If no NOAA station exists within 15 miles, escalate to Environmental Manager for manual approval of a more distant station with documented justification. |
| Secondary station distance | Must be within 25 miles of the site centroid. May be the same type as primary (NOAA COOP, ASOS, GHCND) or a site-installed gauge. |
| Site rain gauge | Where installed, always serves as either primary or secondary. Gauge readings do not replace NOAA data — they supplement it. Both sources are stored. |
| Station type preference | ASOS (airport automated) > COOP (cooperative observer) > GHCND (historical daily) > site gauge. ASOS stations report hourly and have the highest data reliability. |
| Minimum per site | Two station assignments required (primary + secondary). System enforces this as a validation rule on site configuration. |

## B.2 Data Fallback Sequence

When precipitation data is needed for a compliance decision, the system follows this fallback order:

| Priority | Source | Condition to Use | Evidence Weight |
| --- | --- | --- | --- |
| 1 | Primary NOAA station | Data available for the date in question | Highest — court-certifiable via NCEI |
| 2 | Site rain gauge | Primary NOAA data unavailable or gauge provides higher temporal resolution needed for the decision | High — site-specific, but requires calibration records |
| 3 | Secondary NOAA station | Primary station and gauge both unavailable | Medium — distance from site may reduce precision |
| 4 | NWS radar estimate | All station data unavailable; radar data pulled from NWS api.weather.gov gridpoint | Low — modeled, not measured. Acceptable only as corroboration, not sole source. |
| 5 | Manual escalation | All automated sources failed | See Section D (Outage Fallback SOP). Requires manual documentation and supervisor sign-off. |

The system records which source was used for every precipitation decision. If a lower-priority source is used, the reason the higher-priority source was unavailable is stored as part of the evidence record.

# C. Override & Dismissal Documentation Rules

Every human decision that overrides or supplements the automated system must be documented with a reason code, free-text justification, and the user’s identity and timestamp. The system will not allow a bare dismiss or a bare manual activation.

## C.1 Alert Dismissal (Threshold Met, No Sampling Activated)

When the automated system generates a rain event alert and the Supervisor dismisses it without activating sampling, the following fields are mandatory:

| Field | Required | Example |
| --- | --- | --- |
| dismiss_reason_code | Mandatory — from controlled dropdown |  |
| NO_DISCHARGE | Field confirmed no discharge at outfall | "Tech 4 drove to site, outfall dry despite 0.6” recorded at Beckley" |
| STATION_ERROR | Station data believed erroneous | "Beckley ASOS reported 1.2”, but all adjacent stations show 0.0”. Suspected sensor malfunction." |
| LOCALIZED_EVENT | Rain at station did not affect site area | "Storm cell tracked north of permit area per radar. Site gauge reads 0.0”." |
| BELOW_ACTUAL | Site gauge shows rainfall below threshold | "NOAA station reads 0.55” but site gauge reads 0.32” (below 0.50” threshold)." |
| OTHER | Free text required (minimum 50 characters) | Must include enough detail for a third party to understand the basis for dismissal |
| dismiss_justification | Mandatory free text, minimum 50 chars | Narrative explanation supporting the reason code |
| dismissed_by | Auto-populated from auth session | Cannot be overridden manually |
| dismissed_at | Auto-populated server-side timestamp | Cannot be overridden manually |

## C.2 Manual Event Declaration (No Automated Threshold Met)

When a Supervisor or Environmental Manager manually declares a rain event without the automated threshold being triggered, the following fields are mandatory:

| Field | Required | Purpose |
| --- | --- | --- |
| manual_trigger_reason_code | Mandatory dropdown |  |
| GAUGE_ONLY | Site gauge exceeded threshold but NOAA station did not | Localized rainfall not captured at NOAA station |
| VISUAL_DISCHARGE | Field personnel observed discharge during or after precipitation | Discharge observed before NOAA data available (e.g., same-day event) |
| RADAR_INDICATED | NWS radar shows significant precipitation over site area | Preemptive dispatch before NOAA daily summary is published |
| PERMIT_REQUIREMENT | Permit language requires sampling under conditions not fully captured by automated thresholds | Edge case coverage for permit-specific trigger language |
| manual_trigger_justification | Free text, minimum 50 chars | Must explain what evidence the declaration is based on |
| supporting_evidence_ids | Array of document/photo UUIDs | Attach gauge readings, radar screenshots, field photos |

HARD RULE: The audit_log entry for any dismiss or manual declaration is immutable. It cannot be edited or deleted after creation. If a correction is needed, a new audit entry is created referencing the original with an explanation of the correction.

# D. API Outage & Data Outage Fallback SOP

If the automated data pipeline cannot retrieve precipitation data, the system must force a documented manual path. The team is never left stuck with no process — they escalate through a defined sequence.

## D.1 Outage Detection

The nightly sync Edge Function (sync-precipitation-data) implements the following outage detection:

- HTTP failure: If the NOAA CDO API returns a non-200 status or times out (30-second timeout), the function retries 3 times with exponential backoff (30s, 60s, 120s).

- Empty data: If the API returns 200 but no precipitation records for a station that normally reports daily, this is flagged as STATION_DATA_GAP (distinct from confirmed zero rainfall).

- Stale data: If no new reading has been received for a station in 48+ hours, the station is marked STALE and a notification is sent to the Environmental Manager role.

- Full API outage: If all stations fail for the same sync cycle, the system generates a SYSTEM_WEATHER_OUTAGE alert to the Supervisor and Environmental Manager roles.

## D.2 Manual Fallback Procedure

When automated data is unavailable, the following manual procedure is mandatory:

| Step | Action | Detail | Responsible |
| --- | --- | --- | --- |
| 1 | Check NOAA website directly | Access weather.gov or ncei.noaa.gov/cdo-web manually. If data is available on the website but API is down, screenshot the data page and upload to system. | Supervisor |
| 2 | Check site rain gauge | If site gauge is installed, read and photograph the gauge. Enter reading manually into system with source_type = 'manual_entry'. | Field Tech / Supervisor |
| 3 | Check NWS radar archive | Pull quantitative precipitation estimate (QPE) from water.noaa.gov for the site coordinates. Screenshot and upload. | Supervisor |
| 4 | Manual weather entry | If no official source is available, Supervisor enters best available estimate with source_type = 'manual_entry' and attaches all supporting documentation (commercial weather app screenshots, local observer reports, field observations). | Supervisor |
| 5 | Document the outage | System auto-generates an outage incident record. Supervisor must add: (a) which sources were checked, (b) what data was/was not available, (c) what decision was made about rain-event sampling. | Supervisor |
| 6 | Backfill when available | When the API recovers, the next sync cycle attempts to backfill the missed date(s). If backfilled data contradicts the manual decision, the Environmental Manager reviews and documents the discrepancy. | Env. Manager (review) |

## D.3 Conservative Default

HARD RULE: When in doubt during a data outage, sample. The cost of collecting an unnecessary rain-event sample is trivial compared to the cost of a missed sampling event under the Consent Decree. If the Supervisor cannot determine whether a qualifying event occurred and cannot reach the Environmental Manager for guidance, the default action is to dispatch the Float tech for sampling.

# E. Evidence Packet — Mandatory Fields

Every rain event — whether it results in sampling, a no-discharge finding, or a dismissed alert — produces an evidence packet. The following fields are mandatory and the system will not allow a rain event record to be closed without them.

| # | Field | Source | Mandatory? |
| --- | --- | --- | --- |
| 1 | Source station ID(s) | Auto from weather_stations | Yes — always |
| 2 | Station name(s) and type(s) | Auto from weather_stations | Yes — always |
| 3 | Raw data pull timestamp | Auto from precipitation_readings.fetched_at | Yes if automated; manual entry timestamp if manual |
| 4 | Rainfall amount (inches) | From precipitation_readings.rainfall_inches | Yes — always |
| 5 | Reading date and time window | From precipitation_readings | Yes — always |
| 6 | Data quality flags | From NOAA API response | Yes if available; “none reported” if not |
| 7 | User who confirmed/declared event | Auto from auth session | Yes — always |
| 8 | Confirmation/declaration timestamp | Auto server-side | Yes — always |
| 9 | Affected outfalls list | From sampling_schedules join | Yes — always |
| 10 | Dispatch timestamp | From notification sent_at | Yes if sampling activated; N/A if dismissed |
| 11 | Sampling completion status per outfall | From sampling_events | Yes — for each affected outfall: completed, NODI, or missed with reason |
| 12 | Sample collection timestamps | From sampling_events.sample_date/time | Yes if sampling occurred |
| 13 | Lab results received status | From lab_results join | Yes once lab turnaround complete |
| 14 | Exception/dismissal notes | From dismiss or manual fields | Yes if alert dismissed or event declared manually |
| 15 | Recurrence interval (if calculated) | From NOAA Atlas 14 lookup | Only for exemption claims |
| 16 | 48-hour sampling proof | From sampling_events within window | Only for exemption claims |
| 17 | Exemption approval chain | From approval_history | Only for exemption claims |

PDF export format: The evidence packet exports as a single PDF containing all mandatory fields, attached precipitation data screenshots or printouts, field photos, and the sampling event record. The PDF includes a generated-on timestamp, the user who generated it, and a document hash for integrity verification. This packet is stored in Supabase Storage (precipitation-evidence bucket) and linked to the precipitation_events record.

# F. Schema Additions Summary

The following new tables and columns are required to implement this module. No existing tables are modified.

## F.1 New Tables

| Table | Columns | Purpose |
| --- | --- | --- |
| weather_stations | 14 | Registry of NOAA stations and site rain gauges with station IDs, coordinates, type, data source, and API endpoint |
| site_weather_station_assignments | 7 | Maps sites to their primary and secondary weather stations with distance and assignment audit trail |
| precipitation_readings | 11 | Individual precipitation measurements from any source (NOAA API, manual entry, gauge upload) with raw API response storage for auditability |

## F.2 New Column on Existing Table

| Table | Column | Type | Purpose |
| --- | --- | --- | --- |
| sampling_schedules | rain_event_trigger | jsonb | Stores trigger thresholds per outfall (rainfall_threshold_inches, trigger_window_hours, discharge_required, field_confirmation_required, max_sample_delay_hours, recurrence_interval_years) |

## F.3 New Edge Function

| Function | Schedule | Purpose |
| --- | --- | --- |
| sync-precipitation-data | Daily 6 AM ET | Pulls previous day’s precipitation from NOAA CDO for all active stations, stores in precipitation_readings, checks thresholds against rain-event outfalls, generates alerts. Implements retry logic and outage detection per Section D. |

## F.4 New Storage Bucket

| Bucket | Purpose |
| --- | --- |
| precipitation-evidence | Stores generated evidence packet PDFs, uploaded gauge photos, radar screenshots, and any manual documentation. RLS scoped by tenant/organization. |

Document Control

| Version | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | April 3, 2026 | Brian Lewis | Initial implementation specification with all four operational controls locked down |